Technical Data Sheet for Genelock®

Molecular Sample Preservation and transport System

US Patents 6,458,546, 759,342 other patents pending.

- Sample should be added to added to Genelock® tube within 10 minutes after collection. Sample should be well expressed in tube by rolling swab tip against

the inside of the tube while in the Genelock chemistry.

- Sample should be in contact with Genelock at least 10 minutes before

specimen processing.

- Once Sample is added to the Genelock®,  there is no need to refrigerate

or freeze the specimen. Low target specimens will be protected.

- Molecular targets protected by the Genelock® include DNA, RNA, mRNA              and gene transcripts. Individual targets may need to be optimized to chemistry        due to inherent target labiality and endogenous tissue chemistry.

- Molecular targets should be protected for at least 7 days up to+ 25°C.

- Genelock® is designed to inactivate or eliminate a broad range of

amplification inhibitors present in a broad range of sample types.

- Genelock® is compatible with all nucleic amplification platforms and

DNA, RNA, mRNA extraction protocols. Individual target validation may be necessary.

Note: The Genelock® device is designed to protect both Prokaryote,

and Eukaryote samples

Genelock is designed to protect multiple DNA/RNA targets in a single sample and has been optimized for multiplex applications.

For Further information please Call Sierra Molecular Corp.technical service at

(209)536-0886

Sierra Molecular Inc

Sonora, CA. 95370

Version 2.0  valid 17 March 2010